Reversing Manoeuvres

Common to all manoeuvres are a POM routine at the start, observations throughout and controlling the speed of the car. Finish with parking brake on and in neutral

Turn In the Road

- As move away full lock right.  Half a metre from kerb counter steer back to straight

- Stop and handbrake

- POM

- Reverse, full lock left. Half a metre from kerb counter steer back to straight

- Stop and handbrake

- POM

- Turn to right as required

- Repeat if more than 3 turns required

Reversing Left

- As passing junction look for a reference point to indicate point where curb turns

- Stop 2 car lengths and parallel to curb

- Straight reverse until rear wheel reaches first point where curb turns (small mirror)

- Turn wheel ¾ to left

- If rear wheel is about to hit curb straighten wheel

- As rear wheel clears where it would have hit back to ¾

- When approaching straight curb of new road check big mirror, when parallel straighten wheel

- Make adjustments as required to get closer/away from curb

Parallel Park

- Position ½ metre from and parallel to car (stop when front seats level with bonnet)

- Reverse until rear wheels of both cars are level

- Full lock turn to left

- Continue until a shallow angle is achieved (less than 45 degrees) then straighten wheel

- When rear wheel is a drain width from curb full lock right (small mirror)

- When parallel to curb straighten wheel

Bay Park

- Position car parallel to bays at least 1 metre away

- Choose bay and count 3 lines from bay (line 1 is closest line of bay going into) For Guildford test centre 2 lines

- Position door handle level with line 3. For Guildford test centre back seat

- Full lock (check small mirrors to check going into bay)

- Straighten wheel when parallel to bay lines

- Finish with all wheels inside bay